Smart Content Delivery System – Design Document

Team: [Aditi Rowjee, PES1UG25CS030], [Seema Haragapuri, PES1UG25], [Aarush, SRN]

GitHub: https://github.com/aditiisherern/smart-cdn-sdn

1. What we built

Clients download files from a virtual IP, 10.0.0.100, which no real server has. A Ryu SDN controller picks a server for each new connection and rewrites the packet addresses so the request goes to that server, and rewrites the replies so the client only ever sees 10.0.0.100. If a server crashes or a link goes down, the controller notices and sends new traffic somewhere that works.

2. Architecture

Ryu controller

/    |      |     \        (OpenFlow)

clients --- s1 -- s2 --- s4 --- servers

h11-h16      \--- s3 ---/       h1-h3 (10.0.0.1-3, port 8080)

- Topology (topology/cdn_topo.py): 4 switches in a square, so there are 2 paths between clients and servers. Links between switches are 10 Mbps with 5 ms delay.

- Server and client (servers/server.py, servers/client.py): Python TCP sockets.

- Controller (controller/cdn_controller.py): decides the server and the path, installs flow rules.

- Link monitor (monitor/link_monitor.py): reads port stats every 2 s.

- Failure commands (experiments/failures.py): kill a server, cut or slow a link.

3. How it works

Sockets: The client sends GET <filename> and the server replies OK <size> followed by the file, or ERR NOT_FOUND / ERR BAD_REQUEST. The server starts a thread per client. Both sides use a 5 s timeout and the client retries once. Each download is logged to servers/client_log.csv with the time taken.

SDN: The controller answers ARP requests itself (including for 10.0.0.100), so broadcasts don't loop around the square. It learns the topology with LLDP and finds paths with networkx. When a client's first packet to 10.0.0.100:8080 arrives, it picks a server and installs flow rules on the path: the first switch changes the destination to the real server, and the last switch on the way back changes the source back to 10.0.0.100. If a link goes down, the flows are deleted and reinstalled on the other path.

Socket–SDN integration: Every 2 s each server sends a UDP message to the controller with how many clients it is serving. The controller keeps this in server_status and picks the least loaded server that is alive. If a server stops sending for 5 s, it is marked dead and not used.

There are three modes in config.py: smart (above), roundrobin, and static (always server 1, no rerouting) to compare against.

4. Design decisions

- Controller answers ARP instead of flooding: the plain learning switch flooded forever on our looped topology and every ping was lost.

- Address rewriting at the edge switches: clients don't need to know about the servers at all.

- Flow rules per connection: every new download gets load balanced, and old rules time out after 10 s.

- Load reports go through the network to 10.0.0.100: Mininet hosts can't reach the controller on 127.0.0.1.

- All IPs, MACs and ports are in config.py so every file uses the same values.

5. Testing

tests/run_d1_tests.py runs all of these automatically and saves the results in tests/d1_results.md:

- pingall between all hosts: no packet loss.

- Download a 1 MB file from 10.0.0.100: works, and the rewrite rules show up in dump-flows.

- Missing file, bad request, unreachable server: correct error, no crash.

- Kill a server while a client is downloading: controller marks it dead, other servers take over.

- Cut the link in use: traffic moves to the other path.

- 6 clients at once: all downloads work and are spread over the 3 servers.

6. Setup and running

We used Ubuntu. Run everything from the project folder.

Install:

sudo apt install -y mininet openvswitch-switch iperf git

git clone https://github.com/aditiisherern/smart-cdn-sdn

cd smart-cdn-sdn

Install Ryu: On Ubuntu 22.04: pip install ryu eventlet==0.30.2 networkx. Ryu doesn't work on Python 3.12 (Ubuntu 24.04), so there we used uv to get Python 3.9:

curl -LsSf https://astral.sh/uv/install.sh | sh

uv venv --python 3.9 .venv-ryu

VIRTUAL_ENV=.venv-ryu uv pip install "setuptools<58" wheel eventlet==0.30.2

VIRTUAL_ENV=.venv-ryu uv pip install --no-build-isolation ryu==4.34 networkx

Run: First clear any old Mininet run with sudo mn -c (this also stops a running controller, so do it before starting one):

sudo mn -c

Then start the controller in its own terminal (on 24.04 use .venv-ryu/bin/ryu-manager):

ryu-manager --observe-links controller/cdn_controller.py monitor/link_monitor.py

Then in a second terminal, either run all the tests:

sudo python3 tests/run_d1_tests.py

or start the network and try it by hand (wait about 5 s first):

sudo python3 topology/cdn_topo.py

mininet> pingall

mininet> h11 python3 servers/client.py --file test_1m.bin

mininet> kill_server h1

mininet> link_down s1 s2

mininet> sh ovs-ofctl -O OpenFlow13 dump-flows s1

If Mininet says "File exists", run sudo mn -c, start the controller again (mn -c stops it too), and try again.